|  | Highway Paving, Barrier, Curb and Sidewalk Operations (For related operations such as steel erection, structural concrete, precast concrete, site work, utilities or other highway construction operations please refer to Inspection and Verification Records which contain those applicable sections.) Installation and Verification Record |
| --- | --- |

PLEASE READ CAREFULLY; This is being provided to you as a tool that you may wish to use in providing a safety and risk management program. You are responsible for providing safety and risk management services. We at Chubb are providing this tool to you in support of our underwriting objectives and we hereby disclaim any obligation to oversee or monitor the adherence to required or otherwise reasonable safety and risk control practices. We further disclaim liability for claims or suits relating to the alleged or actual failure to conduct reasonable safety control practices, or relating to the use of or failure to use this document. Please note that neither this tool or any of its components contain every possible operation, inspection requirement or other detail that may be relevant to your project or that may be required by federal or state law, or by local building codes, statutes, or other requirements. This tool is intended only as a guideline, and not as a substitute for consultation with your insurance broker, or for engineering, legal or other professional advice.

| Project Identifier: |  | CM/GC: |  |  |
| --- | --- | --- | --- | --- |
| Owner: |  | Date: |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
| Subcontractor Management | Prior to start of operations, verify the following are in place at a minimum for each selected subcontractor: |  |  |  |  |  |  |  |  |
|  | - Subcontractor pre-qual process that includes at a minimum: review of last 3 yrs. Experience Modification Rate (EMR), OSHA violations, losses, financial status and safety culture review/assessment. |  |  |  |  |  |  |  |  |
|  | - Signed/executed subcontract agreement. |  |  |  |  |  |  |  |  |
|  | - Contract includes hold harmless and indemnification agreements in your favor (to the extent allowable by law). |  |  |  |  |  |  |  |  |
|  | - Insurance requirements and limits, to include completed operations coverage, meet contract specifications. |  |  |  |  |  |  |  |  |
|  | - Includes primary and non-contributory additional insured status for contractor, its officer/employees, owner and any others required. |  |  |  |  |  |  |  |  |
|  | - Verify valid certificates of insurance and endorsements are in place. |  |  |  |  |  |  |  |  |
|  | - An appropriately rated insurance carrier. |  |  |  |  |  |  |  |  |
|  | - Signed waiver of subrogation for WC/GL is received. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
|  | - Where required, verify contractor is licensed to work in/for the city and state where project is located. |  |  |  |  |  |  |  |  |
| Approved Shop Drawings and Specifications | - Build in strict compliance with approved specifications and shop drawings. |  |  |  |  |  |  |  |  |
|  | - Stamped approvals from all applicable parties including building department, Dept. of Transportation (D.O.T.), structural engineer and/or architect. |  |  |  |  |  |  |  |  |
|  | - If shop drawings are required, NO portion of work should be commenced until approved by building department, D.O.T., structural engineer and/or architect. |  |  |  |  |  |  |  |  |
|  | - Rfi’s (request for information) are monitored and followed through to verify completion. |  |  |  |  |  |  |  |  |
|  | - Continuous monitoring is conducted as plans change throughout course of project. |  |  |  |  |  |  |  |  |
|  | - Confirm communication / coordination of drawings/specifications between applicable contractors such as steel fabricator and erector, concrete supplier and contractor, underground utilities, utility suppliers and companies, electrical, etc. |  |  |  |  |  |  |  |  |
|  | - Constructability reviews are completed to identify conflicts, verify adequacy of specifications and ensure the projects is designed and built for the climate it is located in. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
| Maintenance and Protection of Traffic (MPT) | The following is performed, at a minimum, for operations where there is the need to provide temporary traffic control. At a minimum, the Manual on Uniform Traffic Control Devices (MUTCD) and all applicable local and state requirements are taken into account. |  |  |  |  |  |  |  |  |
|  | - Traffic control plan (TCP) is created for all ops requiring traffic control that meets or exceeds the MUTDC and any additional local and state requirements. |  |  |  |  |  |  |  |  |
|  | - The TCP is prepared and approved by someone who is authorized and trained and/or certified in Temporary Traffic Control (TTC). |  |  |  |  |  |  |  |  |
|  | - The TCP is communicated to all parties involved in work zone operations (to include local emergency services, law enforcement and railroads). |  |  |  |  |  |  |  |  |
|  | - If traffic control specs are provided by state DOT or other governing authority, a review is conducted by trained/certified persons to ensure plans are appropriate for the construction activities and environment. |  |  |  |  |  |  |  |  |
|  | - If TTC plans become insufficient, they must be immediately reviewed and modified accordingly. |  |  |  |  |  |  |  |  |
|  | - If changes to the TCP are warranted, they are approved by persons who are knowledgeable in proper TTC practices, such as someone who is trained and/or certified in TTC before implemented. |  |  |  |  |  |  |  |  |
|  | - TTC set up, tear down and maintenance operations are preplanned, written and formally communicated to all parties required to perform and oversee these tasks. |  |  |  |  |  |  |  |  |
|  | - TTC set up, maintenance and tear down operations are preplanned/designed to, prevent workers and road users from occupying the same roadway space at the same time. |  |  |  |  |  |  |  |  |
|  | - If workers and road user must occupy same roadway space, precautions are taken to separate them using traffic control devices, lane closures, physical barriers, attenuator vehicles or other safe means. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
| Maintenance and Protection of Traffic (MPT) (Continued) | - Once the work zone is established, a routine inspection program is implemented to include documented discrepancies, corrective actions taken and the use of photos/videos to memorialize proper set up. |  |  |  |  |  |  |  |  |
|  | - If manual flagging operations are to be performed, only trained/certified flaggers are used. |  |  |  |  |  |  |  |  |
|  | - Ensure proper flagging equip. is provided such as; warning signs, paddles, comm. devices, lighting (if necessary), flags (emergency only), retro-reflective outer garments, hardhats etc. |  |  |  |  |  |  |  |  |
|  | - If MPT is subcontracted, steps must be taken ensuring TTC is set up, maintained and torn down in accordance with contract specifications, the MUTDC and any applicable local and state requirements. |  |  |  |  |  |  |  |  |
| Survey Operations | Surveys performed for location, design, construction, maintenance and operation of projects. The following are conducted at a minimum: |  |  |  |  |  |  |  |  |
|  | - Qualified survey crew available in advance of progressing operations. |  |  |  |  |  |  |  |  |
|  | - A mark is useless if it is destroyed or lost. Marks are located in areas protected from damage caused by construction operations, general public, erosion, etc. |  |  |  |  |  |  |  |  |
|  | - Marks must be well described (visible) for future recovery. |  |  |  |  |  |  |  |  |
|  | - Instrument records must be created for each instrument used on the project. |  |  |  |  |  |  |  |  |
|  | - Stake-outs establishing basic line and grade for project construction are performed just prior to actual construction. |  |  |  |  |  |  |  |  |
|  | - All baseline of construction controls are referenced with rebar, spikes, cross-cuts in concrete or other permanent markers. |  |  |  |  |  |  |  |  |
|  | - Verify that all survey notes/data are of good quality (readable). Notes must be easily understood, free from ambiguity, confusion, uncertainty, and doubt. |  |  |  |  |  |  |  |  |
|  | - Field notes are complete. They have been field checked, contain all applicable elements, built in self-checks, and required qualities. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
|  | - All survey data (instrument recorders, Personal Digital Assistants (PDA), field logs/notes, drawings etc.) are turned over to the project engineers for review and retention. |  |  |  |  |  |  |  |  |
| Cranes | - Are either mobile and/or tower cranes to be used to facilitate these operations? |  |  |  |  |  |  |  |  |
|  | - If so, has the ACE “Crane Installation and Verification Record” section been completed? |  |  |  |  |  |  |  |  |
| Equipment/Material Delivery | - Assure all equipment & materials necessary for highway related work is visually inspected to verify compliance with final approved specifications and shop drawings and is damage/defect free upon arrival. |  |  |  |  |  |  |  |  |
|  | - Field modifications are not conducted unless approved by all parties including the structural engineer and the owner in writing. |  |  |  |  |  |  |  |  |
| Welding Operations | Welding Operations are monitored and at a minimum the following are completed: |  |  |  |  |  |  |  |  |
|  | - Verify welders are trained and qualified in accordance with local, state and federal standards and codes. |  |  |  |  |  |  |  |  |
|  | - Document & verify welder’s qualifications. |  |  |  |  |  |  |  |  |
|  | - Verify a “Hot Work” program is in place that covers fire protection (extinguishers, fire watch), permit and training requirements. |  |  |  |  |  |  |  |  |
|  | - At a minimum, all welds are inspected on a continuous, on-going basis for strength, quality and completeness. |  |  |  |  |  |  |  |  |
| Reinforced Concrete Paving Inspections | Inspections to verify compliance w/ approved drawings and specifications are completed. Inspections include but are not limited to: |  |  |  |  |  |  |  |  |
|  | - Construction as per final approved shop drawings. |  |  |  |  |  |  |  |  |
|  | - Concrete testing. |  |  |  |  |  |  |  |  |
|  | - Stripping operations. |  |  |  |  |  |  |  |  |
|  | - Concrete design mix. |  |  |  |  |  |  |  |  |
|  | - Reinforcing steel. |  |  |  |  |  |  |  |  |
|  | - Curing. |  |  |  |  |  |  |  |  |
|  | - Finish. |  |  |  |  |  |  |  |  |
|  | - Wedge bolts. |  |  |  |  |  |  |  |  |
|  | - Expansion/control joints. |  |  |  |  |  |  |  |  |
|  | - Rebar/formwork clearance. |  |  |  |  |  |  |  |  |
| Concrete Design Mix | Concrete mixes that are proportioned, mixed, or batched are inspected to verify compliance w/ approved drawings and specifications both on-site and from information obtained from the batch plant. Data includes but is not limited to: |  |  |  |  |  |  |  |  |
|  | - Strength (psi). |  |  |  |  |  |  |  |  |
|  | - Water/cement ratio. |  |  |  |  |  |  |  |  |
|  | - Date. |  |  |  |  |  |  |  |  |
|  | - Water amount. |  |  |  |  |  |  |  |  |
|  | - Cement amount. |  |  |  |  |  |  |  |  |
|  | - Slump. |  |  |  |  |  |  |  |  |
|  | - Laboratory licensing information. |  |  |  |  |  |  |  |  |
|  | - Laboratory analysis data. |  |  |  |  |  |  |  |  |
|  | - Amount of fine and course aggregate. |  |  |  |  |  |  |  |  |
|  | - Admixtures such as air-entraining, strength enhancements, accelerators, decelerators, or any other ingredient added to concrete to alter its properties. |  |  |  |  |  |  |  |  |
|  | - Verification and sign-off from architect and or engineer. |  |  |  |  |  |  |  |  |
| Precast Concrete Barriers | - Ensure barrier meets approved standards and specifications and are inspected upon delivery (height, length, design). |  |  |  |  |  |  |  |  |
|  | - Barrier secured using required fasteners, bolts, plates or other as required by approved specifications and drawings. |  |  |  |  |  |  |  |  |
|  | - Barrier is secure at proper intervals as per approved specifications and drawings. |  |  |  |  |  |  |  |  |
| Utility Installation and Relocations | Prior to installing new, relocating or removing existing overhead and/or underground utilities, the following items and inspections at a minimum, are conducted: |  |  |  |  |  |  |  |  |
|  | - Obtain & review most current utility installation drawings. Reviewed by field engineers for accuracy. |  |  |  |  |  |  |  |  |
|  | - Verify that “call before you dig” or “one call” system has been requested to perform mark out of existing underground utilities. |  |  |  |  |  |  |  |  |
|  | - Request mark out from privately owned utility Co’s not part of the local call system. |  |  |  |  |  |  |  |  |
|  | - Maintain record of utility mark out requests. Use photos to memorialize m/o locations in the event of a mismark, marks removed or destroyed or strike occurs. |  |  |  |  |  |  |  |  |
|  | - Compare mark out locations with utility specs/drawings to identify potential conflicts. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
| Utility Installation and Relocations (Continued) | - If conflicts are identified, they are reviewed by field engineer & new design and/or installation specs determined. |  |  |  |  |  |  |  |  |
|  | - Ensure methods to safely excavate underground utilities are used such as hand digging, vacuum systems and non-conductive equipment used. |  |  |  |  |  |  |  |  |
|  | - During backfill ops, assure underground utilities are not crushed or damaged. If necessary, additional designed support systems are used fro protection. |  |  |  |  |  |  |  |  |
|  | - Overhead utilities are de-energized & grounded or relocated by the utility provider. |  |  |  |  |  |  |  |  |
|  | - Verify & document overhead utilities are de-energized & grounded, relocated or otherwise protected from damage. |  |  |  |  |  |  |  |  |
| Fill Material | Fill material inspections are conducted to verify compliance w/ approved drawings and specifications. Inspections include, but are not limited to: |  |  |  |  |  |  |  |  |
|  | - Use of test pits for uncontrolled fill. |  |  |  |  |  |  |  |  |
|  | - Proper field density tests per lift. |  |  |  |  |  |  |  |  |
|  | - Proper environmental testing. |  |  |  |  |  |  |  |  |
|  | - Proper composition and size of materials. |  |  |  |  |  |  |  |  |
|  | - Proper clean fill certifications. |  |  |  |  |  |  |  |  |
|  | - Check for satisfactory bearing material. |  |  |  |  |  |  |  |  |
|  | - Confirm ground surface preparation. |  |  |  |  |  |  |  |  |
|  | - Proper compacting, watering and layering. |  |  |  |  |  |  |  |  |
|  | - Allowable soil bearing pressure. |  |  |  |  |  |  |  |  |
|  | - Inspections for voids and inclusions. |  |  |  |  |  |  |  |  |
|  | - Optimum moisture content. |  |  |  |  |  |  |  |  |
|  | - Implementation of grading controls. |  |  |  |  |  |  |  |  |
| Soils Compaction | - Verify proper compacting, watering and layering. |  |  |  |  |  |  |  |  |
|  | - At a minimum, the following areas should be focused on: perimeter walls, retaining walls, street improvements (sewer, water and storm drains). |  |  |  |  |  |  |  |  |
|  | - Re-compaction conducted at locations that have been excavated for piping & utilities. |  |  |  |  |  |  |  |  |
|  | - Soil compaction meets applicable DOT standards, contract specifications. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
| Paving Base and Sub Base | Inspections to verify compliance w/ approved drawings and specifications are completed. Inspections include but are not limited to: |  |  |  |  |  |  |  |  |
|  | - Asphalt base is constructed on a prepared foundation that has been constructed according to approved standards and specifications. |  |  |  |  |  |  |  |  |
|  | - Ensure placement of base has proper depth and number of lifts. |  |  |  |  |  |  |  |  |
|  | - Ensure shaping and compacting of the base to product uniform texture and firmly key the aggregates. |  |  |  |  |  |  |  |  |
|  | - Subgrade modifications performed per approved standards and specs. These may include: cleaning, reshaping ditches, scarifying and compacting subgrade, realigning shoulders. |  |  |  |  |  |  |  |  |
|  | - Reconditioning of the road surface is performed according to approved standards and specifications. This may include: scarifying roadbed, pulverizing scarified material, sifting to assure proper size and compaction. |  |  |  |  |  |  |  |  |
|  | - Specialty bases courses such as Lime and Concrete treated are constructed according to approved standards and specifications. |  |  |  |  |  |  |  |  |
|  | - If the Contractor is directed by the authority having jurisdiction to open to traffic any portion of new pavement and/or structures before final acceptance of the contract is completed, this direction is in writing and maintained in the job files. |  |  |  |  |  |  |  |  |

|  | Inspection Criteria |  |  |  | Responsible Party(ies) |  |  | Inspected by |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category | Description | Yes | No | N/A | Installation Contractor | Contracted with | Comments/Notes | Initial | Date |
| Asphalt Paving | Prior to and during paving operations, inspections to verify compliance w/ approved drawings and specifications are completed. Inspections include but are not limited to: |  |  |  |  |  |  |  |  |
|  | - Verify delivered asphalt mixture meets or exceeds approved standards and specification required for the project. |  |  |  |  |  |  |  |  |
|  | - Verify hauling vehicles are properly covered, insulated, and a release agent is used to allow removal of the mix. |  |  |  |  |  |  |  |  |
|  | - Proper base coat is established by repairing the surface where necessary and applying a tack coat to all contacting surfaces. |  |  |  |  |  |  |  |  |
|  | - Weather and temperature requirements are acceptable at time of paving. |  |  |  |  |  |  |  |  |
|  | - Spread and finish the mixture with asphalt pavers to specified grade and thickness maintaining a consistent supply of mixture to ensure uninterrupted paving. |  |  |  |  |  |  |  |  |
|  | - Asphalt paving equipment is equipped with required devices such as activated heated screed assemblies and automatic screed controls. |  |  |  |  |  |  |  |  |
|  | - Compact immediately after spreading and before the asphalt mixture falls below the minimum job-mix design compaction temperature. |  |  |  |  |  |  |  |  |
|  | - Discontinue paving if unable to achieve the specified density before the mixture cools to below specified temperatures. |  |  |  |  |  |  |  |  |
|  | - Construct a control strip with production materials and equipment. If the job-mix formula, the compacting method, or compacting equipment changes, or if non-specification results occur, reconstruct the test strip. |  |  |  |  |  |  |  |  |
|  | - Ensure compliance with job mix requirements with random core samples. |  |  |  |  |  |  |  |  |
|  | - Obtain random test samples from the compacted pavement for the full depth of the course. |  |  |  |  |  |  |  |  |
|  | - Suspend paving if paving tolerances are not in compliance with approved standards and specifications. Resume only when the situation is corrected. |  |  |  |  |  |  |  |  |
| Concrete Paving Concrete Paving (Continued) | Inspections to verify compliance w/ approved drawings and specifications are completed. Inspections include but are not limited to: |  |  |  |  |  |  |  |  |
|  | - Ensure the approved mix design proportions, strength and restrictions meet approved standards and specifications or submit a mix design proposal for approval. |  |  |  |  |  |  |  |  |
|  | - If contractor designed mix is to be used, conduct trial batch testing of the proposed mix. |  |  |  |  |  |  |  |  |
|  | - If contractor designed mix is to be used, submit test results showing compliance with specified minimum strength, durability, air content, aggregate gradation limits, and workability for approval. |  |  |  |  |  |  |  |  |
|  | - If contractor designed mix is to be used, use laboratory tests to determine the cement content required with individual aggregate combinations. |  |  |  |  |  |  |  |  |
|  | - If contractor designed mix is to be used, establish exact proportions by testing trial mixes and adjust to produce concrete that meets plasticity and workability requirements. |  |  |  |  |  |  |  |  |
|  | - If contractor designed mix is used, provide a new mix design, based on trial mixes, for approval when there is any change in the source, character of materials used during production of concrete for the project. |  |  |  |  |  |  |  |  |
|  | - Ensure that the batch plant site, layout, equipment, and arrangements for transporting the material will provide a continuous concrete supply. |  |  |  |  |  |  |  |  |
|  | - If furnishing batch plant mixers, ensure they are capable of thoroughly combining aggregates, cement, and water & discharge the mixture. Equip. is equipped with timing device automatically preventing discharge during mixing & allows discharge only when mixing is finished. |  |  |  |  |  |  |  |  |
|  | - Reject concrete mixed less than the specified time. |  |  |  |  |  |  |  |  |
|  | - Furnish truck mixers for mixing & hauling concrete & agitators for hauling central-mixed concrete that meet or exceed approved plans and specifications. |  |  |  |  |  |  |  |  |
|  | - Provide vibrators to consolidate the concrete for the full width. |  |  |  |  |  |  |  |  |
|  | - If using Slip-Form Method. Place concrete with a slip-form paver that can spread, consolidate, screed, and finish the freshly placed concrete with one complete pass. |  |  |  |  |  |  |  |  |
|  | - When forms are used ensure they meet requirements such as length, thickness of metal, are as deep as pavement edge is thick and can be kept rigid during paving. |  |  |  |  |  |  |  |  |
|  | - Stop mixing and concreting operations if shaded ambient air temperature away from artificial heat is less than specified temperatures. |  |  |  |  |  |  |  |  |
|  | - Resume operations only when the ambient air temperature is at or above specified temperatures and rising. |  |  |  |  |  |  |  |  |
|  | - Place concrete only on unfrozen subgrade or ensure that no frozen aggregate is in the concrete mix. |  |  |  |  |  |  |  |  |
|  | - Longitudinal, contraction and expansion joints are placed per approved standards and specifications. |  |  |  |  |  |  |  |  |
|  | - Correct surface irregularities while the concrete is plastic, after completing floating and removing excess water and laitance. |  |  |  |  |  |  |  |  |
|  | - If surface texture is required, texture the surface without tearing it after the surface sheen disappears. |  |  |  |  |  |  |  |  |
|  | - Ensure concrete is cured for the minimum specified duration immediately after the finishing operation. |  |  |  |  |  |  |  |  |
|  | - Ensure concrete is protected for the minimum specified duration or until concrete achieves a minimum compressive strength specified in approved specifications and drawings. |  |  |  |  |  |  |  |  |
|  | - Protect the concrete from freezing with blankets, or other materials, when the temperature drops below specified tolerances. Remove and replace all concrete damaged by freezing. |  |  |  |  |  |  |  |  |
|  | - Cover and cure entire surface and edges using approved methods such as impervious membrane, burlap, waterproof paper and polyethylene film. |  |  |  |  |  |  |  |  |

|  |  | Inspection Criteria |  |  |  |  |  |  |  | Responsible Party(ies) |  |  |  |  |  | Inspected by |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category |  | Description |  | Yes |  | No |  | N/A |  | Installation Contractor |  | Contracted with |  | Comments/Notes |  | Initial |  | Date |
| Curb, Sidewalks and Other Walking Surfaces | - Ensure all curb, sidewalk, ramps etc. are constructed according to approved standards and specifications and are in compliance with the American’s with Disabilities Act (ADA). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Proper sub base is established before curb/sidewalk concrete is poured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Required expansion/control joints are in place. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Ensure required/proper finish is applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Curing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value Engineering | If a value engineering program is available and will be used the following items need to be considered at a minimum: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Specs/instructions regarding the value engineering program are obtained. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Value engineering specs are read and understood to ensure compliance. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - There are complete plans and specifications, which meet standards, showing the proposed changes relative to the original contract features and requirements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - The contractor requires a professional engineer’s stamp and signature on any significant engineering changes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Full descriptions of differences btwn existing contract requirements & proposed changes, & comparative advantages & disadvantages of each (i.e. considerations of service life, economy of operation, ease of maintenance, traffic flow, safety, desired appearance, progress schedule & any increase/reduction of environmental impacts are submitted). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Any value engineering proposal is not acted upon until final approval by the authority responsible for the program. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Documentation of approved value engineering proposals are obtained and maintained in job files. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | Inspection Criteria |  |  |  |  |  |  |  | Responsible Party(ies) |  |  |  |  |  | Inspected by |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspection/Test Category |  | Description |  | Yes |  | No |  | N/A |  | Installation Contractor |  | Contracted with |  | Comments/Notes |  | Initial |  | Date |
| Turn Over Information | - The following items, at a minimum, are provided to the project owner, Govt. agency etc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Training for owners, maintenance staff, etc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Construction documents such as drawings, specifications and blueprints. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Warranty information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Maintenance and service instructions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Installation design plans. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Any training products and resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Location of utilities and structures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - All punch list items and inspection report deficiencies addressed and corrected to the satisfaction of the inspection agency and/or manufacturer? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - Document record retention program in place and per specifications. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

VERIFICATION STATEMENT I verify that to the best of my knowledge, the inspections and/or tests set out in this installation and verification record were completed by the responsible party shown, that any recommended corrections were made, and that any reports accompanying these inspections/tests were received for review, file and sign off and (if required) PE/RE stamps.

Name (please print in full)

Initials Date

Chubb is the marketing name used to refer to subsidiaries of Chubb Limited, providing insurance and related services. For a list of these subsidiaries, please visit our website, www.chubb.com. Insurance is provided by ACE American Insurance Company and its U.S. based Chubb underwriting company affiliates. All products may not be available in all states. This communication contains product summaries only. Coverage is subject to the language of the policies as actually issued. Surplus lines insurance is sold only through licensed surplus lines producers. Loss control evaluations, reports, recommendations and services are made solely to assist the insurer in underwriting and loss control and are not to be construed as an added benefit for the insured, property owner or any other party (this may not apply if loss control services are purchased separately and specifically pursuant to a service agreement). Evaluation for any hazard or condition does not imply that it is covered under any policy. Chubb is the world’s largest publicly traded property and casualty insurance group. With operations in 54 countries, Chubb provides commercial and personal property and casualty insurance, personal accident and supplemental health insurance, reinsurance and life insurance to a diverse group of clients. Chubb Limited, the parent company of Chubb, is listed on the New York Stock Exchange (NYSE: CB) and is a component of the S&P 500 index. Copyright ©2016